# EEPROM Editor User Manual

## 1. Overview

The EEPROM Editor is the kiosk’s maintenance interface for configuring non-volatile settings and service data.

Primary use cases:

- Commissioning and HWID setup.

- Hydraulic and kiosk parameter tuning.

- NFC token table management.

- RTC date/time setting.

- Service counters inspection/reset.

The editor is a boot-time service mode and is non-returning. Exit is by reset.

## 2. Entry And Exit

## 2.1 How To Enter

You can enter EEPROM Editor in either way:

- Automatic maintenance entry:

  - On boot, if EEPROM magic is invalid or HWID is 0.

- Manual boot chord:

  - Hold Sensor Bypass + Backwash during boot.

  - Release when prompted.

## 2.2 How To Exit

- Long BACK (~4s) triggers watchdog reset.

- Re-init EEPROM flow ends with reset.

- Editor does not return directly to runtime flow.

## 3. High-Level Navigation Diagram

EEPROM Editor Navigation

## 4. Buttons In EEPROM Editor Mode

During editor mode, panel buttons are remapped:

| Editor Action | Physical Button Constant | Typical Panel Label |
| --- | --- | --- |
| UP | BTN_CONT_CIRC | Continuous Circulation |
| DOWN | BTN_WATER_INLET | Water Inlet |
| SELECT | BTN_SENSOR_BYPASS | Sensor Bypass |
| BACK | BTN_BACKWASH_CTL | Backwash |

Press semantics:

- Short UP/DOWN: move cursor or change value.

- Hold UP/DOWN: auto-repeat starts after ~2s, then accelerates.

- Short SELECT: enter submenu, advance field, or toggle prompt.

- Long SELECT (~3s): save/confirm for current context.

- Short BACK: go to previous menu layer.

- Long BACK (~4s): reset and exit editor.

Special BACK behavior:

- In RTC Config, short BACK moves to previous RTC field first.

- In NFC Add/Del scan screens, short BACK exits immediately.

## 5. Save/Write Behavior Diagram

EEPROM Editor Save Model

## 6. Display Layout (OLED)

General layout:

- Row 0: EEPROM Edit Mode

- Row 1: separator

Top-level menu only:

- Row 2: HWID:...

- Row 3: SWID:...

Lower-level menus:

- Row 2: current menu title

- Row 3: blank

- Menu list starts at row 4

Value presentation:

- For value-heavy layers (e.g. Backwash, Water Disp, NFC Params),

  - row 12 is cleared,

  - current value is centered on row 13.

- Row 14 is normally blank for editor layers.

Status line:

- Row 15 shows one of:

  - Saving....

  - Saved !!

  - navigation hint

RTC screen is a special layout:

- Date/time display lines are shown with a caret row indicating active editable cluster.

## 7. Top-Level Menu Structure

Top-level entries:

- PCB HW ID

- Dispensed Units

- Kiosk Parm Edit

- Hydra Parm Edit

- NFC Token Mgt

- Re-init EEPROM

### 7.1 Kiosk Parm Edit

- Solenoid Config

- KlaranUV Config

- WaterTempSensor

- CoinAcc Config

- RTC Config

- NFC Params Mgt

### 7.2 Hydra Parm Edit

- Backwash Config

- SensBypass Edit

- Water Circ Ctrl

- Water Disp Ctrl

- Periodic Flush

### 7.3 NFC Token Mgt

- Browse/Delete

- NFC Add Token

- NFC Del Token

- Pack NFC Table

- Del ALL Tokens

## 8. Menu Details

## 8.1 PCB HW ID

Purpose:

- Set stored HWID used by firmware/profile branching.

Operation:

- UP/DOWN change HWID.

- Long SELECT saves.

## 8.2 Dispensed Units

Menu entries:

- App Disp

- Coin Disp

- NFC Token Disp

- Bypassed Disp

- TOTAL Dispensed (read-only aggregate)

Operation:

- First four counters are editable.

- Long SELECT saves edited counter.

## 8.3 Kiosk Parm Edit

### 8.3.1 Solenoid Config

For each solenoid profile:

- StartPWM

- HoldPWM

- SwOnDelay (seconds)

### 8.3.2 KlaranUV Config

- UV OK Delay (ms)

- UV MAX ON time (mins)

### 8.3.3 WaterTempSensor

- Toggle T1/T2.

### 8.3.4 CoinAcc Config

- Toggle fitted state YES/NO.

### 8.3.5 RTC Config

Editable clusters in sequence:

- YYYY, MM, DD, HH, MM, SS

Controls:

- UP/DOWN: adjust active cluster (wrap behavior applied).

- Short SELECT: write current RTC value and move to next cluster.

- Short BACK: move to previous cluster; from first cluster, exit RTC screen.

- Long SELECT: explicit save still available.

Important behavior:

- On entry, fields are pre-populated from DS3231.

- Date/time is clamped to valid ranges before write.

### 8.3.6 NFC Params Mgt

- NFC Init Delay (ms)

- NFC Scan Duratn (ms)

- Inter NFC Delay (ms)

## 8.4 Hydra Parm Edit

### 8.4.1 Backwash Config

Parameters:

- Auto duration (s)

- Auto after N (pulse)

- Backwash dispense counter (pulse)

- Daily duration (s/mins)

- Daily time (HH:MM)

- Manual short (s)

- Manual long (mins)

### 8.4.2 SensBypass Edit

Parameters:

- Duration (s)

- Period (mins)

- Manual short (s)

- Manual long (s)

### 8.4.3 Water Circ Ctrl

Parameters:

- Auto duration (mins)

- Auto period (mins)

- Manual short (mins)

- Manual long (mins)

### 8.4.4 Water Disp Ctrl

Parameters:

- Dispense mode (TIMED / PULSE)

- Timed duration (s)

- Pulse count (pulse)

- Pre-dispense circulation/purge time and pulse values

### 8.4.5 Periodic Flush

Parameters:

- DNF repeat period (mins)

- DNF duration (s)

- Pre-flush beep delay (s)

- Beep period (ms)

- Beep on-time (ms)

- Beep count

## 8.5 NFC Token Mgt

### 8.5.1 Browse/Delete

Display format:

- I ### <HASH>

- I indicates table index.

Navigation:

- UP: decrement index

- DOWN: increment index

- Wrap at first/last slot.

Delete flow:

- Short SELECT: enter delete prompt (YES/NO).

- Long SELECT on YES: delete selected token.

### 8.5.2 NFC Add Token

Flow:

- Screen waits for tag.

- Two consecutive matching scans are required.

- On success, token hash is added.

### 8.5.3 NFC Del Token

Flow:

- Same two-scan validation.

- On success, matching token hash is removed.

### 8.5.4 Pack NFC Table

- Compacts and de-duplicates token table.

### 8.5.5 Del ALL Tokens

- Opens confirmation modal.

- Toggle YES/NO.

- Long SELECT executes delete-all when confirmed.

## 8.6 Re-init EEPROM

Safety flow:

- Enter prompt.

- Set confirmation to YES.

- Hold SELECT for hold-time countdown.

Result:

- EEPROM reset to defaults.

- Resetting/completed messages shown.

- Device restarts.

## 9. Typical Service Workflows

## 9.1 Set RTC Quickly

- Kiosk Parm Edit -> RTC Config.

- Edit field with UP/DOWN.

- Short SELECT to write and advance.

- Repeat for all fields.

- BACK out when done.

## 9.2 Add NFC Token

- NFC Token Mgt -> NFC Add Token.

- Present tag until success message.

- BACK to return to token menu.

## 9.3 Tune Backwash Auto Trigger

- Hydra Parm Edit -> Backwash Config.

- Edit BW AutoAfter N.

- Long SELECT to save.

- Confirm status line shows save message.

## 10. Troubleshooting

- No DS3231 RTC !

  - RTC not detected. Check DS3231 wiring, power, and I2C lines.

- NFC add/delete not changing table

  - Verify tag is scanned twice consistently.

  - Verify token writes are permitted in the editor configuration.

- Edited values revert

  - Ensure long SELECT save was executed (except RTC next-field writes).

- Cannot leave editor

  - Use long BACK to force reset.

## 11. Safety And Change Control

- Re-init EEPROM is destructive. Use only with planned recovery.

- Record critical values (RTC, backwash, dispense, NFC timing) before major changes.

- Perform configuration updates in controlled maintenance windows.