| Category | Variable | EEPROM Address | Type | Stored Range | Units / LSB | Effective Range | Reset Default (raw) | Reset Default (effective) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core | HWID | 0 | uint8 | 0..255 | raw | 0..255 | 0 | 0 | Hardware ID |
| Dispense Profile | Measured Duration | 1-2 | uint16 | 0..65535 | 100ms/LSB | 0..6553.5s | 200 | 20.0s | ADDR_DISP_DURATION100MS |
| Dispense Profile | Measured Pulses | 3-4 | uint16 | 0..65535 | pulses | 0..65535 | 2000 | 2000 | ADDR_DISP_PULSES |
| Dispense Profile | Dispense Mode | 5 | uint8 | 0..255 | enum | 0=time,1=pulses | 0 | TIME | ADDR_DISP_MODESEL |
| Solenoid BackWash | Start PWM | 10 | uint8 | 0..255 | PWM | 0..255 | 255 | 255 |  |
| Solenoid BackWash | Hold PWM | 11 | uint8 | 0..255 | PWM | 0..255 | 130 | 130 |  |
| Solenoid BackWash | SW Delay | 12 | uint8 | 0..255 | seconds | 0..255s | 5 | 5s |  |
| Solenoid BackWash | Reserved0 | 13 | uint8 | 0..255 | raw | 0..255 | 0 | 0 | legacy |
| Solenoid BackWash | Reserved1 | int8 @14 | int8 | -128..127 | raw | -128..127 | 0 | 0 | legacy |
| Solenoid SenseByp | Start PWM | 15 | uint8 | 0..255 | PWM | 0..255 | 255 | 255 |  |
| Solenoid SenseByp | Hold PWM | 16 | uint8 | 0..255 | PWM | 0..255 | 130 | 130 |  |
| Solenoid SenseByp | SW Delay | 17 | uint8 | 0..255 | seconds | 0..255s | 5 | 5s |  |
| Solenoid SenseByp | Reserved0 | 18 | uint8 | 0..255 | raw | 0..255 | 0 | 0 | legacy |
| Solenoid SenseByp | Reserved1 | int8 @19 | int8 | -128..127 | raw | -128..127 | 0 | 0 | legacy |
| Solenoid Inlet | Start PWM | 20 | uint8 | 0..255 | PWM | 0..255 | 255 | 255 |  |
| Solenoid Inlet | Hold PWM | 21 | uint8 | 0..255 | PWM | 0..255 | 130 | 130 |  |
| Solenoid Inlet | SW Delay | 22 | uint8 | 0..255 | seconds | 0..255s | 5 | 5s |  |
| Solenoid Inlet | Reserved0 | 23 | uint8 | 0..255 | raw | 0..255 | 0 | 0 | legacy |
| Solenoid Inlet | Reserved1 | int8 @24 | int8 | -128..127 | raw | -128..127 | 0 | 0 | legacy |
| Solenoid Dispense | Start PWM | 25 | uint8 | 0..255 | PWM | 0..255 | 128 | 128 |  |
| Solenoid Dispense | Hold PWM | 26 | uint8 | 0..255 | PWM | 0..255 | 100 | 100 |  |
| Solenoid Dispense | SW Delay | 27 | uint8 | 0..255 | seconds | 0..255s | 5 | 5s |  |
| Solenoid Dispense | Reserved0 | 28 | uint8 | 0..255 | raw | 0..255 | 0 | 0 | legacy |
| Solenoid Dispense | Reserved1 | int8 @29 | int8 | -128..127 | raw | -128..127 | 0 | 0 | legacy |
| Backwash | Deprecated BW Freq | 30 | uint8 | 0..255 | deprecated | unused | 0 | 0 | reserved/unused |
| Backwash | BW AutoDuration | 31 | uint8 | 0..255 | 5s/LSB | 0..1275s | 0 | disabled | 0 disables default-duration backwash |
| Backwash | BW AutoAfter N | 32 | uint8 | 0..255 | count | 0..255 | 0 | 0 | 0 disables |
| Backwash | BW Disp Counter | 33-36 | uint32 | 0..4294967295 | count | 0..4294967295 | 0 | 0 |  |
| KlaranUV | UV OK Delay | 37 | uint8 | 0..255 | 10ms/LSB | 0..2550ms | 150 | 1500ms |  |
| KlaranUV | UV MAX ON Time | 38 | uint8 | 0..255 | minutes | 0..255m | 60 | 60m |  |
| General | Water Temp Sensor | 39 | uint8 | 1..2 (clamped) | enum | 1=T1,2=T2 | 1 | T1 | invalid values clamp to 1 |
| NFC Timing | NFC Init Delay | 40 | uint8 | 10..250 (clamped) | 1ms/LSB | 10..250ms | 100 | 100ms | NFC timing defaults |
| NFC Timing | NFC Scan Duration | 41 | uint8 | 1..250 (stored), 10..2500ms API | 10ms/LSB | 10..2500ms | 15 | 150ms |  |
| NFC Timing | NFC Inter Delay | 42 | uint8 | 10..250 (clamped) | 1ms/LSB | 10..250ms | 50 | 50ms |  |
| Backwash Daily | Daily BW Time | 43-44 | uint16 | 0..1439 (clamped) | minutes since midnight | 00:00..23:59 | 60 | 01:00 |  |
| Backwash Daily | Daily BW Duration | 45 | uint8 | 0..255 | 10s/LSB | 0..2550s | 0 | 0s | 0 disables |
| General | Coin Acceptor Fitted | 46 | uint8 | 0..1 | bool | 0/1 | 0 | false |  |
| Legacy RTC stamp metadata | Daily BW Year Offset | 47 | uint8 | 0..225 | year-2000 | 2000..2225 | 0 | 0 | legacy |
| Legacy RTC stamp metadata | Trig BW Year Offset | 48 | uint8 | 0..225 | year-2000 | 2000..2225 | 0 | 0 | legacy |
| Legacy RTC stamp metadata | Daily BW Month | 49 | uint8 | 1..12 | month | 1..12 | 0 | 0 | legacy |
| Legacy RTC stamp metadata | Trig BW Month | 50 | uint8 | 1..12 | month | 1..12 | 0 | 0 | legacy |
| SensorBypass | SensByp Duration | 51 | uint8 | 0..255 | 100ms/LSB | 0..25.5s | 0 | 0s | 0 disables |
| SensorBypass | SensByp Period | 52 | uint8 | 0..255 | minutes | 0..255m | 0 | 0m | 0 disables |
| Dispense | Disp Avg Count | 53 | uint8 | 1..100 (clamped) | count | 1..100 | 10 | 10 |  |
| Water Circ | AutoCirc Duration | 54 | uint8 | 0..255 | minutes | 0..255m | 0 | 0m | 0 disables |
| Water Circ | AutoCirc Period | 55 | uint8 | 0..255 | 10 minutes/LSB | 0..2550m | 0 | 0m | 0 disables |
| Water Circ | ManCirc Short | 56 | uint8 | 0..255 | minutes | 0..255m | 1 | 1m | 0 disables |
| Water Circ | ManCirc Long | 57 | uint8 | 0..255 | 10 minutes/LSB | 0..2550m | 1 | 10m | 0 disables |
| Dispense | PreDispCircT | 58 | uint8 | 0..255 | seconds | 0..255s | 0 | 0s |  |
| Dispense | PreDispCircP | 59 | uint8 | 0..255 | 100 pulses/LSB | 0..25500 pulses | 0 | 0 |  |
| Dispense | PreDispPurgT | 60 | uint8 | 0..255 | seconds | 0..255s | 0 | 0s |  |
| Dispense | PreDispPurgP | 61 | uint8 | 0..255 | 100 pulses/LSB | 0..25500 pulses | 0 | 0 |  |
| Backwash | BW MAN Short | 62 | uint8 | 0..255 | seconds | 0..255s | 15 | 15s |  |
| Backwash | BW MAN Long | 63 | uint8 | 0..255 | 2 minutes/LSB | 0..510m | 15 | 30m |  |
| SensorBypass | MAN Bypass Short | 64 | uint8 | 0..255 | 100ms/LSB | 0..25.5s | 50 | 5.0s |  |
| SensorBypass | MAN Bypass Long | 65 | uint8 | 0..255 | 10s/LSB | 0..2550s | 6 | 60s |  |
| Periodic Flush | DNF Rept Period | 66 | uint8 | 0..255 | minutes | 0..255m | 60 | 60m |  |
| Periodic Flush | DNF DispDurat | 67 | uint8 | 0..255 | 100ms/LSB | 0..25.5s | 50 | 5.0s |  |
| Periodic Flush | PreFlshBeepDely | 68 | uint8 | 0..255 | seconds | 0..255s | 15 | 15s |  |
| Periodic Flush | Beep ON Time | 69 | uint8 | 0..255 | 10ms/LSB | 0..2550ms | 5 | 50ms |  |
| Periodic Flush | Beep Period | 70 | uint8 | 0..255 | 100ms/LSB | 0..25500ms | 10 | 1000ms | nearest to requested 950ms |
| Periodic Flush | Beep Count | 71 | uint8 | 0..255 | count | 0..255 | 0 | 0 | 0 disables periodic flush |
| Core | Magic | 124-127 | uint32 | 0..4294967295 | signature | 0x4D4A4343 | 0x4D4A4343 | layout marker |  |
| Dispensed Counters | DISP_APP | 128-131 | uint32 | 0..4294967295 | count | 0..4294967295 | 0 | 0 |  |
| Dispensed Counters | DISP_COIN | 132-135 | uint32 | 0..4294967295 | count | 0..4294967295 | 0 | 0 |  |
| Dispensed Counters | DISP_NFC | 136-139 | uint32 | 0..4294967295 | count | 0..4294967295 | 0 | 0 |  |
| Dispensed Counters | DISP_BYPASS | 140-143 | uint32 | 0..4294967295 | count | 0..4294967295 | 0 | 0 |  |
| NFC Tokens | Token hash slot 000 | 3072-3075 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x0A5D281B | 0x0A5D281B | Default token hash |
| NFC Tokens | Token hash slot 001 | 3076-3079 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x0D1193C2 | 0x0D1193C2 | Default token hash |
| NFC Tokens | Token hash slot 002 | 3080-3083 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x2022301E | 0x2022301E | Default token hash |
| NFC Tokens | Token hash slot 003 | 3084-3087 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x356BB66D | 0x356BB66D | Default token hash |
| NFC Tokens | Token hash slot 004 | 3088-3091 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x4800E934 | 0x4800E934 | Default token hash |
| NFC Tokens | Token hash slot 005 | 3092-3095 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x4DBEB28A | 0x4DBEB28A | Default token hash |
| NFC Tokens | Token hash slot 006 | 3096-3099 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x591AC9A8 | 0x591AC9A8 | Default token hash |
| NFC Tokens | Token hash slot 007 | 3100-3103 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x5CC8C061 | 0x5CC8C061 | Default token hash |
| NFC Tokens | Token hash slot 008 | 3104-3107 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x7F026686 | 0x7F026686 | Default token hash |
| NFC Tokens | Token hash slot 009 | 3108-3111 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x841709BF | 0x841709BF | Default token hash |
| NFC Tokens | Token hash slot 010 | 3112-3115 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x84AEC09C | 0x84AEC09C | Default token hash |
| NFC Tokens | Token hash slot 011 | 3116-3119 | uint32 | 0..4294967295 | hash | 32-bit hash | 0x9A9FEE82 | 0x9A9FEE82 | Default token hash |
| NFC Tokens | Token hash slot 012 | 3120-3123 | uint32 | 0..4294967295 | hash | 32-bit hash | 0xE2963725 | 0xE2963725 | Default token hash |
| NFC Tokens | Token hash slot 013 | 3124-3127 | uint32 | 0..4294967295 | hash | 32-bit hash | 0xE3557210 | 0xE3557210 | Default token hash |
| NFC Tokens | Token hash slots 014-255 | 3128-4095 | uint32 each | 0..4294967295 | hash | 32-bit hash | 0x00000000 | 0x00000000 | Cleared by default |
| NFC Tokens | Address basis |  | text |  |  | tokenBase = EEPROM.length() - 1024 bytes (256 x uint32) |  |  | For Arduino Mega 2560 this resolves to 3072 |