# Kiosk Software Operating Manual

## 1. Scope

This manual describes the runtime software behavior of the kiosk firmware in Kiosk.ino and its primary modules:

- KioskBehavior.*

- KioskHydraulics.*

- KioskIO.*

- KioskEeprom.* (runtime parameter reads/writes)

Explicitly excluded from this manual:

- EEPROM Editor mode (KioskEepromEditor.*)

- Sequential I/O Test mode (KioskSeqTest.*)

Those have separate dedicated manuals.

## 2. Runtime Architecture

The runtime loop is split into three control layers:

- UI layer (KioskBehavior::Ui)

- Runs the LCD-facing customer state machine.

- Emits requestDispensePulse and LED/backlight intents.

- Policy/orchestration layer (KioskBehavior::updateHydraulics)

- Resolves mode-specific rules and button choreography.

- Arbitrates SensorBypass, Backwash, WaterInlet, Tank circulation, DayEnd, and Periodic Nozzle Flush.

- Actuator layer (KioskHydraulics + KioskIO)

- Executes dispense/UV/backwash low-level state machines.

- Drives physical outputs and inlet PWM kick/hold behavior.

## 3. Operating Modes

## 3.1 Normal Mode

- Payment is required before dispense.

- Front button drives payment and dispense flow via UI states.

- Rear hydraulic control buttons are still read by hydraulics policy.

## 3.2 Override Mode

- Front button bypasses payment path (goes directly to UI_DISP_RDY).

- Hydraulic control buttons can override/terminate conflicting functions (as defined below).

## 3.3 Dark Mode

- Enter from UI_WELCOME using the front-button gesture: press in Override, then release after the mode switch is moved to Normal.

- Entry phase (UI_DARK_MODE_IN) shows a 60s countdown; pressing front can truncate to immediate dark transition.

- In dark phase, LCD backlight and front LED are off; OLED row 14 shows IN DARK MODE.

- Exit phase (UI_DARK_MODE_OUT) requires holding front button for 15s.

- Releasing early during exit returns to dark-in state.

- Completing exit hold requests a reset.

## 4. Boot-Time Entry Points

At startup the firmware checks button chords:

- Dispense calibration mode: BTN_SINGLE_DISP held at boot.

- EEPROM editor chord (excluded from this manual).

- Sequential test chord (excluded from this manual).

## 5. UI State Machine

The runtime UI state machine is shown below.

UI Runtime State Machine

### 5.1 UI State Summary

| State | Purpose | Main exit conditions |
| --- | --- | --- |
| UI_BOOT | Show counters on LCD | Front press/release or 60s timeout |
| UI_WELCOME | Idle welcome screen | Normal: to payment; Override: to dispense ready; dark gesture; out-of-service |
| UI_PAY_PROMPT | Payment collection | Payment OK/failed/bad token, or 15s timeout |
| UI_PAY_*_OK | Payment success acknowledgements | 2s to UI_DISP_RDY |
| UI_PAY_FAILED | Payment failed notice | Front press retry or 15s timeout |
| UI_PAY_TOKEN_BAD | Unknown token notice | Front press or 1s timeout |
| UI_DISP_RDY | Waiting for dispense trigger | Front/rear dispense press, or 60s timeout |
| UI_DISPENSING | Active dispense progress | Dispense complete |
| UI_DISP_DONE | Dispense complete message | Front press or 2s timeout |
| UI_TIMEOUT | Transaction timeout message | 3s timeout |
| UI_OUT_OF_SERVICE | Not currently dispensable | Auto-return when dispensable again |
| UI_DARK_MODE_IN | Dark-mode entry and dark hold state | Front press in dark -> UI_DARK_MODE_OUT |
| UI_DARK_MODE_OUT | Dark-mode exit hold | 15s hold -> reset, or release -> UI_DARK_MODE_IN |
| UI_ERROR | Error state | Self-refresh loop until reset/recovery path |

## 6. Hydraulics Core State Machine

Low-level dispense, UV gating, and backwash execution are implemented in KioskHydraulics.

Hydraulics Core State Machine

### 6.1 Dispense and UV Interlock

Dispense state flow:

- IDLE -> UV_WAIT on dispense edge if:

- water level > 0

- selected dispense profile is enabled

- UV_WAIT (100ms):

- If UV_OK is HIGH, transition to DISPENSE.

- If UV_OK is LOW, abort and latch ERR_UV_NOT_OK.

- DISPENSE:

- UV remains enabled.

- If UV_OK goes LOW at any time, dispense aborts and ERR_UV_NOT_OK is latched.

- Completion is by selected mode: time target or pulse target.

- UV_POST (100ms) after dispense completion:

- UV remains on for post-run delay.

- Then returns to IDLE.

### 6.2 Backwash Overlay

Backwash runs in parallel with dispense state:

- Start request while idle:

  - Normal: requires water level > 1

  - Override: requires water level > 0

- Request while active: toggles backwash OFF.

- Auto-stop on duration expiry or tank level < 1.

## 7. Hydraulics Orchestration Rules

KioskBehavior::updateHydraulics() handles policy and conflict resolution.

## 7.1 Water Level Semantics

- 0: empty

- 1: low

- 2: mid

- 3: full (FULL sensor active-low asserted)

## 7.2 Water Inlet Strategy

Inlet demand combines an auto latch and a manual latch:

- Startup prime:

- On first update after boot, if level < 3, manual inlet demand is set ON.

- Hysteresis latch:

- If level < 2 -> auto latch ON.

- If level > 2 -> auto latch OFF.

- At level == 2 -> hold previous auto latch state.

- Hard safety:

- If level > 2, manual inlet demand is cleared.

- Physical inlet output is forced OFF.

- Manual hold-off (BTN_WATER_INLET):

- If inlet demand is currently ON, press enters hold-off for up to 1 hour.

- If OFF/HOLD and level < 3, press sets manual inlet demand ON and clears hold.

- Hold auto-expires after 1 hour.

- Override suppression cancel:

- In Override, inlet button press can terminate active SensorBypass and/or Backwash first.

- If that suppression was blocking inlet and level < 3, inlet demand is re-enabled immediately.

- Final inlet ON condition:

- (autoLatch OR manualLatch) AND !hold AND level < 3 AND !backwashActive AND !sensorBypass.

## 7.3 SensorBypass

- Manual toggle source: BTN_SENSOR_BYPASS release.

- Duration chosen by press length:

  - <= 2s -> MAN Bypass Short (100ms LSB)

  - > 2s -> MAN Bypass Long (10s LSB)

- Auto expiry by timer.

- If periodic bypass is enabled, schedule is maintained and restartable from button interaction.

Interaction with Backwash:

- Normal mode:

  - If backwash active/starting, SensorBypass request is queued.

- Override mode:

  - If backwash active, button first stops backwash then applies bypass toggle.

## 7.4 Backwash

- Manual source: BTN_BACKWASH_CTL release.

- Manual duration chosen by press length:

  - <= 2s -> BW MAN Short (1s LSB)

  - > 2s -> BW MAN Long (2min LSB)

- Second manual press while active stops backwash.

- Auto sources:

  - Daily RTC schedule (BW Daily Time, BW Daily Duratn)

  - Dispense-count trigger (BW AutoAfter N)

Daily trigger rules:

- RTC must be present.

- Only in the first 60 minutes after configured daily time.

- Daily duration must be non-zero.

- Not already run for the same calendar date.

- Requires normal backwash eligibility by mode and level.

Dispense-count trigger rules:

- On each completed dispense: increment BW DispCount.

- If BW AutoAfter N == 0: auto trigger disabled.

- If count reaches threshold:

  - reset count to 0

  - queue auto backwash request

Backwash completion also resets BW DispCount to 0.

## 7.5 Tank Circulation

Manual circulation is controlled by BTN_CONT_CIRC:

- Press while inactive and level > 0:

- Start circulation immediately.

- Final timeout is resolved on button release.

- Release:

- <= 2s press -> short duration profile (ManCircDur Shrt, minutes LSB).

- > 2s press -> long duration profile (ManCircDur Long, 10-minute LSB).

- Effective timeout subtracts already-held press time.

- Press while active:

- Immediately terminates circulation and resets active tracking.

Auto circulation:

- Enabled only when both AutoCirc Duratn > 0 and AutoCirc Period > 0.

- Runs periodically at configured period for configured duration (only if level > 0).

Dispense interaction:

- Active dispense pauses circulation output and timer.

- Remaining circulation time is extended by paused duration.

## 7.6 DayEnd Circ+BW

DayEnd sequence is queueable only in Override mode using chord:

- Hold BTN_BACKWASH_CTL

- Hold BTN_CONT_CIRC

- Release BTN_BACKWASH_CTL

- Release BTN_CONT_CIRC

On queue/init:

- Existing circulation/backwash activity is terminated.

- Auto-backwash and pending backwash are cleared.

- Active dispense is aborted.

- Inlet demand is restored if level < 3.

Sequence:

- Run EoD circulation (EoD Circ Durat, 15s LSB), start only when level > 1.

- Then run EoD backwash (EoD BW Durat, 1s LSB), start only when level > 1.

- On completion, DayEnd state clears and inlet demand is restored if level < 3.

DayEnd Circ+BW State Machine

## 7.7 Periodic Nozzle Flush (DNF)

DNF is a timed automatic dispense in UI_WELCOME context.

Enable conditions:

- DNF Rept Period > 0

- DNF DispDurat > 0

- Beep ON Time > 0

- Beep Period > 0

- Beep Count > 0

- Not in dark mode

- Not in DayEnd

- UI currently in UI_WELCOME

Behavior:

- On entering UI_WELCOME, DNF timer and phase are reset/armed.

- After repeat period elapses:

- Acoustic alert pulses run (setAcousticAlert on Ozone output).

- Optional pre-flush delay is applied.

- DNF timed dispense starts when dispense path is free.

- DNF dispense uses DNF-specific timed duration (not normal dispense profile).

- On completion, DNF returns to idle and waits for next repeat period.

Cancellation:

- Leaving UI_WELCOME, entering dark mode/dayEnd, invalid config, or non-periodic dispense activity can cancel pending DNF phases.

Periodic Nozzle Flush State Machine

## 8. Control Buttons

| Button | Normal mode | Override mode |
| --- | --- | --- |
| Front Dispense | UI navigation + payment flow + dispense start in UI_DISP_RDY | UI navigation + direct dispense path (no payment) |
| BTN_SINGLE_DISP (rear single) | Acts as rear dispense trigger in UI_DISP_RDY | Same |
| BTN_BACKWASH_CTL | Request/stop backwash; respects level and SensorBypass hold-off | Can terminate SensorBypass and start backwash if eligible |
| BTN_SENSOR_BYPASS | Toggle bypass; held if backwash active | Can stop backwash first, then toggle bypass |
| BTN_WATER_INLET | Toggle inlet demand/1h hold-off | Same plus suppression-cancel override behavior |
| BTN_CONT_CIRC | Start/stop manual circulation with short/long durations | Same |
| BTN_CONT_DISP | Runtime hydraulics: not used as main control | Used in DispenseCAL mode workflow |

## 9. Displays

## 9.1 Customer LCD (16x4)

Primary content is driven by UI state machine:

- Welcome/payment/dispense progress/timeout/error messages.

- In UI_DISPENSING, row 3 updates at controlled rate (not full-screen redraw every loop).

## 9.2 Rear OLED (16 rows)

Key runtime rows:

- Row 0: mode/state header (Kiosk Operations or dark-state id text).

- Row 2-4: water level, backwash eligibility, dispense eligibility.

- Row 5: inlet PWM phase (PH0/PH1/PH2).

- Row 7: Backwash status (OFF/AUT/ON/EoD) + right-justified counter.

- Row 8: SenseByp status (OFF/AUT/ON) + counter.

- Row 9: WaterInp status (OFF/AUT/ON) + counter.

- Row 10: TankCirc status (OFF/AUT/ON/EoD) + counter.

- Row 11: Dispense status (OFF/AUT/ON/DNF) + counter.

- Row 13/14: Dark-mode and DayEnd centered overlays.

- Row 15: NORM/OVRD + RTC datetime YYMMDDHHMM.

## 10. Error Handling and Fail-Safe Behavior

Error codes:

- 100 = dispense profile unavailable

- 101 = UV not OK during dispense precheck or active dispense

When an error is active:

- Software latches are cleared.

- Backwash, SensorBypass, WaterInlet, Dispense, Circulation, and acoustic alert are forced OFF.

- UI transitions to UI_ERROR (except while already in dark states).

## 11. Runtime EEPROM Parameters Used by Operations

This section lists runtime parameters used by kiosk operation (not editor behavior):

| Feature | Parameters used at runtime |
| --- | --- |
| Dispense | Disp Ctrl Mode, TimedDisp Dur, PulseDisp Count |
| Backwash | BW AutoDuration, BW AutoAfter N, BW MAN Short, BW MAN Long, BW Daily Time, BW Daily Duratn |
| SensorBypass | SensByp Duratn, SensByp Period, MAN Bypass Short, MAN Bypass Long |
| Water inlet solenoid | Inlet solenoid StartPWM, HoldPWM, SwDelaySec |
| Tank circulation | AutoCirc Duratn, AutoCirc Period, ManCircDur Shrt, ManCircDur Long |
| DayEnd | EoD Circ Durat, EoD BW Durat |
| Periodic nozzle flush | DNF Rept Period, DNF DispDurat, PreFlshBeepDely, Beep ON Time, Beep Period, Beep Count |

## 12. Dispense Calibration Mode (Boot Chord)

This mode is entered by holding BTN_SINGLE_DISP at boot.

Purpose: - Measure manual dispense samples from the front button. - Build an average sample duration. - Save average duration to EEPROM measured dispense duration field.

Workflow:

- Press and hold front dispense to start sample flow.

- Release front button to stop sample and enter review.

- In review:

- Short release on front: reject sample.

- Long release on front: accept sample into aggregate.

- Long press on BTN_CONT_DISP or BTN_SINGLE_DISP:

- Save aggregate average to EEPROM.

- Trigger watchdog reset.

UI details:

- LCD row 3 shows sample count (N=) left-aligned and average (AVG=) right-aligned.

- Long-press threshold crossing blinks LCD backlight for 50ms.

## 13. Software/Hardware Notes

- Target platform: Arduino Mega 2560 (arduino:avr:mega).

- Water inlet PWM is two-phase: kick then hold (PH1 -> PH2).

- Acoustic alert output is mapped to Ozone output pin.

- UV_OK runtime logic is active-high (HIGH means UV ready/healthy).