# Sequential I/O Test User Manual

## 1. Purpose

The Sequential I/O Test is a boot-time service mode used to verify:

- Output channels (solenoids, pumps, relays, LEDs, UV/Ozone paths).

- Key digital inputs and switch states.

- Temperature sensor readings.

- Water-level and Klaran UV-OK feedback.

This mode is intended for bench/service diagnostics, not normal kiosk operation.

## 2. Entry And Exit

## 2.1 Entering Sequential I/O Test

At power-up, hold either of these buttons:

- BTN_OZONE_CTL, or

- BTN_CONT_CIRC

When the release prompt appears, release the invocation button(s).

## 2.2 Exiting Sequential I/O Test

Sequential test mode is non-returning and runs continuously.

To exit:

- Reset/power-cycle the controller.

## 3. Mode Flow Diagram

Sequential I/O Test Flow

## 4. Screen Behavior

## 3.1 OLED (primary live diagnostics)

- Row 0: I/O SeqTest Mode

- Row 1: separator

- Rows 2-4: instruction text

- Row 6: active output name (and Klaran timeout countdown when applicable)

- Rows 8-13: live input/status summary lines

## 3.2 LCD (operator header)

- Line 0: I/O SeqTest Mode

- Line 1: separator

- During release prompt: HWID/SWID are shown.

## 5. Controls During Test

The test cycles one output at a time.

- Next output: press any of

  - BTN_OZONE_CTL

  - BTN_CONT_DISP

  - BTN_CONT_CIRC

- Previous output: press

  - BTN_WATER_INLET

Navigation behavior:

- If no output is active, first “next” starts at index 0.

- If no output is active, “previous” starts at the last output.

- Exactly one output is active at a time.

## 6. Output Test Sequence

The sequence depends on HWID.

Shared sequence (all HWIDs):

- Dispense Button LED (PWM_DISP_BTTN_LED)

- Backwash Solenoid (PWM_BACKWASH_SOL)

- Backwash Pump (OUT_BACKWASH_PUMP)

- Water Sensor Bypass Solenoid (PWM_SENSOR_BYP_SOL)

- Inlet Boost Pump (OUT_INLET_BOOST_PUMP)

- Inlet Solenoid PWM (PWM_INLET_SOL)

- Dispense Pump (OUT_WATERDISP_PUMP)

- Dispense Solenoid (PWM_WATERDISP_SOL)

If HWID < 3:

- Klaran UV (OUT_KLARAN_UV)

- External UV (OUT_EXT_UV)

- Ozone (OUT_OZONE)

If HWID >= 3:

- Coin Acceptor (OUT_COIN_ACCEPTOR)

- External Relay 2 (OUT_EXT_RELAY_2)

- External Relay 1 (OUT_EXT_RELAY_1)

- Generic UV (OUT_EXT_UV)

- Klaran UV (OUT_KLARAN_UV)

Notes:

- PWM channels are driven at full duty while active.

- Non-PWM outputs are driven HIGH while active.

- All non-active outputs are forced OFF.

## 7. Output Sequence Diagram

Sequential I/O Output Sequence

## 8. Klaran UV Special Handling

When the active output is Klaran UV:

- A 15-second safety timer starts.

- OLED row 6 shows remaining seconds.

- After timeout, Klaran UV output is forced OFF automatically.

Klaran status line (Klaran OK) behavior:

- N/A when Klaran UV is not currently under active timed check.

- OK when IN_KLARAN_UV_OK input is HIGH during active Klaran window.

- NO when the input is not asserted during active Klaran window.

## 9. Live Input/Status Lines (OLED Rows 8-13)

- SW: 1=... 2=...

  - SW1 uses OVERRIDE_SWITCH

  - SW2 uses IN_SW_2

- Btn: 1=... 2=...

  - Button 1 uses BTN_FRONT_DISPENSE

  - Button 2 uses BTN_SPARE_BUTTON

- FloatSW: Active/Inactive

  - Uses IN_WATER_LEVEL_FULL (active-low)

- Temp 1:xxx 2:yyy

  - From one-wire sensors on IN_TEMP_T1 and IN_TEMP_T2

  - Updates approximately once per second

  - Invalid reads shown as --

- Level: L=.. M=..

  - Raw state of IN_WATER_LEVEL_LOW and IN_WATER_LEVEL_MED

- Klaran OK: ...

  - As described in Section 6

## 10. Typical Test Procedure

- Enter Sequential I/O Test at boot.

- Confirm release prompt and enter main screen.

- Use “next output” buttons to step through every output.

- For each step:

  - Verify expected actuator energizes.

  - Verify all others are off.

- Observe rows 8-13 for switch/input telemetry.

- For Klaran UV step:

  - Confirm 15-second countdown appears.

  - Verify auto shutoff at timeout.

  - Verify Klaran OK response.

- Reset/power-cycle to leave test mode.

## 11. Troubleshooting

- No mode entry at boot:

  - Re-check button hold timing and wiring for BTN_OZONE_CTL / BTN_CONT_CIRC.

- Cannot step outputs:

  - Verify button inputs for BTN_OZONE_CTL, BTN_CONT_DISP, BTN_CONT_CIRC, BTN_WATER_INLET.

- Wrong output activates:

  - Confirm board HWID; sequence differs for <3 vs >=3.

- Temperature shows --:

  - Check one-wire sensors and bus wiring.

- Klaran never shows OK:

  - Check IN_KLARAN_UV_OK signal path and sensor logic.

## 12. Safety Notes

- This mode directly energizes pumps/solenoids/relays one-by-one.

- Use appropriate safe hydraulic/electrical test conditions.

- Keep downstream loads safe before activating high-power outputs.